Co2

2.1

Aim:

To generate and distinguish between periodic and aperiodic signals using MATLAB. The

Aim is to demonstrate the fundamental difference between periodic and aperiodic signals by

creating examples of each and analyzing their properties.

Program

clc;

clear;

close all;

% Time axis

t = -5:0.01:5;

% Periodic Signal (Sine Wave)

x1 = sin(2*pi*t);

% Aperiodic Signal (Exponential Decay)

x2 = exp(-abs(t));

% Plotting

figure;

subplot(2,1,1);

plot(t,x1,'LineWidth',2);

grid on;

title('Periodic Signal');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(2,1,2);

plot(t,x2,'LineWidth',2);

grid on;

title('Aperiodic Signal');

xlabel('Time (s)');

ylabel('Amplitude');

output wave form

Result:

• Periodic Signal: The sinusoidal signal repeats itself at regular intervals, making it

periodic.

• Aperiodic Signal: The exponentially decaying signal does not repeat, showcasing

that it is aperiodic.

2.2

Aim

Decompose a signal into even and odd parts and verify reconstruction.

Program

clc;

clear;

close all;

% Time axis

t = -5:0.1:5;

% Original Signal

x = t.^3 + t;

% Time-reversed Signal

x_rev = (-t).^3 + (-t);

% Even Component

xe = (x + x_rev)/2;

% Odd Component

xo = (x - x_rev)/2;

% Reconstruction of Original Signal

x_reconstructed = xe + xo;

% Plotting

figure;

subplot(4,1,1);

plot(t,x,'LineWidth',2);

grid on;

title('Original Signal x(t)');

xlabel('Time');

ylabel('Amplitude');

subplot(4,1,2);

plot(t,xe,'LineWidth',2);

grid on;

title('Even Component x_e(t)');

xlabel('Time');

ylabel('Amplitude');

subplot(4,1,3);

plot(t,xo,'LineWidth',2);

grid on;

title('Odd Component x_o(t)');

xlabel('Time');

ylabel('Amplitude');

subplot(4,1,4);

plot(t,x_reconstructed,'LineWidth',2);

grid on;

title('Reconstructed Signal x_e(t) + x_o(t)');

xlabel('Time');

ylabel('Amplitude');

output waveform

Result

Signal equals sum of even and odd parts.

2.3

Aim

Demonstrate amplitude and time scaling.

Program

clc;

clear;

close all;

% Time axis

t = -5:0.01:5;

% Original Signal

x = sin(2*pi*t);

% Amplitude Scaling

x_amp = 2*x;

% Time Scaling (Compression)

x_time = sin(2*pi*2*t);

% Plotting

figure;

subplot(3,1,1);

plot(t,x,'LineWidth',2);

grid on;

title('Original Signal x(t)');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(3,1,2);

plot(t,x_amp,'LineWidth',2);

grid on;

title('Amplitude Scaled Signal 2x(t)');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(3,1,3);

plot(t,x_time,'LineWidth',2);

grid on;

title('Time Scaled Signal x(2t)');

xlabel('Time (s)');

ylabel('Amplitude');

outputwaveform

Result

Scaling affects amplitude or time axis accordingly.

2.4

Aim

Demonstrate time shift and reversal.

Program

clc;

clear;

close all;

% Time axis

t = -5:0.01:5;

% Original Signal

x = sin(2*pi*t);

% Time Shift (Delay by 2 seconds)

x_shift = sin(2*pi*(t-2));

% Time Reversal

x_reverse = sin(2*pi*(-t));

% Plotting

figure;

subplot(3,1,1);

plot(t,x,'LineWidth',2);

grid on;

title('Original Signal x(t)');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(3,1,2);

plot(t,x_shift,'LineWidth',2);

grid on;

title('Time Shifted Signal x(t-2)');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(3,1,3);

plot(t,x_reverse,'LineWidth',2);

grid on;

title('Time Reversed Signal x(-t)');

xlabel('Time (s)');

ylabel('Amplitude');

output waveform

Result

The sinusoidal signal was successfully subjected to time shifting and time reversal using

MATLAB. The time-shifted signal was delayed by 2 seconds, while the time-reversed signal

was obtained by reflecting the original signal about the vertical axis, demonstrating

fundamental signal transformation operations.