Aim

To verify the generation of a sinusoidal signal with a specific frequency, amplitude, and phase using MATLAB. The Aim is to ensure that the signal generator can accurately produce a sinusoidal waveform with the desired parameters.

Program: (1.1)

% Generation of Continuous Time Signals

clc;

clear;

close all;

% Time axis

t = -5:0.01:5;

% Continuous-time sine signal

x = sin(2*pi*t);

% Plotting

figure;

plot(t, x, 'LineWidth', 2);

grid on;

title('Continuous Time Sine Signal');

xlabel('Time (seconds)');

ylabel('Amplitude');

Output Waveform:

Result:

This test case validates that the signal generator can produce a sinusoidal waveform with the

specified frequency, amplitude, and phase shift.

1.2

Aim:

To generate and verify a discrete-time unit impulse signal using MATLAB. The Aim is to ensure that the signal generator can accurately produce an impulse signal at a specified timeindex, which is crucial for analyzing and understanding the behavior of discrete-time systems.

Program

clc;

clear;

close all;

% Discrete-time index

n = -10:10;

% Discrete-time sinusoidal signal

x = sin(0.2*pi*n);

% Plotting

figure;

stem(n, x, 'filled');

grid on;

title('Discrete Time Sinusoidal Signal');

xlabel('Sample Index (n)');

ylabel('Amplitude');

output waveform

Result:

This test case validates that the signal generator can accurately produce a discrete-time unitimpulse signal at a specified index, which is essential for analyzing the response of discrete-time systems.

1.3

Aim

Generate and plot triangular and sawtooth signals.

Program

clc;

clear;

close all;

% Time axis

t = 0:0.01:2;

% Sawtooth Signal

x1 = sawtooth(2*pi*t);

% Triangular Signal

x2 = sawtooth(2*pi*t,0.5);

% Plotting

figure;

subplot(2,1,1);

plot(t,x1,'LineWidth',2);

grid on;

title('Sawtooth Signal');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(2,1,2);

plot(t,x2,'LineWidth',2);

grid on;

title('Triangular Signal');

xlabel('Time (s)');

ylabel('Amplitude');

Output Generated Graph

Result

Triangular wave has equal rise and fall; sawtooth shows sharp drop and gradual rise.

1.4

Aim

Generate and plot unit impulse, unit step, and ramp functions.

Program

clc;

clear;

close all;

%% Continuous-Time Signals

t = -5:0.01:5;

% Continuous Unit Step Function

u = double(t >= 0);

% Continuous Ramp Function

r = t .* (t >= 0);

%% Discrete-Time Signals

n = -5:5;

% Discrete Unit Step Function

ud = double(n >= 0);

% Discrete Ramp Function

rd = n .* (n >= 0);

%% Plotting

figure;

subplot(2,2,1);

plot(t,u,'LineWidth',2);

grid on;

title('Continuous Unit Step Function');

xlabel('Time (t)');

ylabel('Amplitude');

subplot(2,2,2);

plot(t,r,'LineWidth',2);

grid on;

title('Continuous Ramp Function');

xlabel('Time (t)');

ylabel('Amplitude');

subplot(2,2,3);

stem(n,ud,'filled');

grid on;

title('Discrete Unit Step Function');

xlabel('Sample Index (n)');

ylabel('Amplitude');

subplot(2,2,4);

stem(n,rd,'filled');

grid on;

outputwaveform